Jane’s Manual for running scenarios in ANUGA

1.	Define the problem area in project.py

- Get map of region of interest (supervisor can sign for free copy at shop)

- Choose problem area and identify latitudes and longitudes

  - Latitude: from 90 deg S to 90 deg N ranges from -90 to 90

  - Longitude: 180 deg W to 180 deg E

  - Update code to read, for example (degrees, minutes, seconds)

    - south = degminsec2decimal_degrees(-34,05,0)

    - north = degminsec2decimal_degrees(-33,45,0)

    - west = degminsec2decimal_degrees(151,0,0)

    - east = degminsec2decimal_degrees(151,30,0)

- Arrange ASCII file from Neil

  - Update basename as supplied to

    - basename = ‘file from Neil renamed to have scenario relevance’

  - xllcorner and yllcorner taken from this file

  - Update mesh_origin as

    - mesh_origin = (grid reference, xllcorner, yllcorner)

    - grid reference from geodesy, 56 for Sydney area

- Define internal regions of interest in project.py

- Set up in the same way as the problem area described above

- Can set up multiple internal regions if desired

- Setting up data files and locations in project.py

- Update directory where results to be written relating to scenario, eg

  - scenario_dir_name = 'sydney_tsunami_scenario_2006'

  - create this directory with the following sub-directories

    - output

    - topographies

    - meshes

- Copy Neil’s file to topographies directory

- Scenario generating file, eg run_sydney.py

- Ensure that interior_regions captures all the internal regions defined in project.py

- Second argument here defines the maximal area of the triangle in that region (units in metres and note that data supplied in eg 100m x 100m grid.)

- Define boundary tags for problem polygon

- Uses a dictionary when creating mesh (boundary_tags, resolution, filename and interior_regions)

  - boundary_tags

    - If points on square numbered anti-clockwise from bottom left from P0 to P3, then boundary segments numbered S0 to S3 (so P0 to P1 is S0). Boundary_tags uses the segment name and the segment number (i.e. not the points).

    - Resolution usually finer in interior_region

- Define initial and boundary conditions in usual way

- IC: set_quantity for friction, stage, elevation

- BC: define boundary conditions (eg Dirichlet (requires value for height, x momentum and y momentum), reflective, transmissive and time) and then associate to boundary tags in dictionary formal.

- Run!

- In domain.evolve, define time to integrate system to and yieldstep defines how often the results are written and visualised

- Debug!