Experiment one:

Aim:

The aim of this first experiment is to initially find friction values for building scenarios that will represent either the best or worst case in terms of reducing momentum of the inundating tsunami. The building scenarios will be performed for a range of building widths in all the layout patterns. It is believed this is required because the hydrodynamic behaviour of each building layout will change as the building dimensions change. These first building scenarios should provide the limiting cases for the friction values that could be applied to that particular building size.

Data produced:

The data produced/required for this experiment is a series of inundation scenarios based on the final domain settings. Friction scenarios have been produced for friction values from 0 to 1 in steps of 0.025 and from 1 to 120 in steps of 1. All 160 of these friction scenarios will be used as comparisons the building scenarios.

The building scenarios as described in the methodology have also been produced for varying building densities/building size. Each of these building scenarios will have a best fit friction value calculated for them. The best fit friction values will be analysed in an attempt to determine what parameters of the building layouts and size control the hydrodynamic behaviour of the inundating wave.

Experiment Limitations and results:

The comparisons of these two scenarios will occur at the 1000 second interval of each simulation. This time interval as been chosen as it is a potential period of the wave height that can occur in a tsunami. It is assumed that both of these scenarios have approached a steady state problem, where the flow in and out of the given scenarios is equal and unchanging over time. At this point it can be assumed that the water levels have achieved their maximum and this will be used to compare the friction and building scenarios. The data recorded at the desired time interval are essentially two independent signals or wavelets. These will be compared using the 2nd norm test, which is a modified Root-Mean-Squared (RMS) test to account for the difference in the two signals relative to the modelled buildings. This test measures the sum of the mean differences between the wavelets and produces a greater then zero result for two non identical signals. The smallest 2nd norm test result will then identify the best match of a friction value to a particular building scenario.

Methodology:

Several building layouts have been chosen to represent both the least and most restrictive layout patterns for a particular building size. All these building layouts have been based on square buildings and square blocks to keep the analysis simple. The chosen layouts are shown below with a brief description of their believed behaviour. Figure 1 (a) shows the straight building or high order (i.e. not random). This layout is believed to provide the least resistance of all the chosen building layouts to the inundating wave. There is a clear path or quasi-channel for water to channel down. The quasi-channels only pinch or cut off flow once the buildings have reached full density. Figure 1 (b) shows an offset of every second row of buildings of half the block length. This places a building in the path of the water channelling between two buildings. This make the flow continually change direction which causes momentum changes and thus energy reductions in the flow. This perfect offset is believed to impart the most momentum changes on the inundating flow. For both figures (a) and (b) the building density can achieve 100% until the buildings will touch each other. Figure 2 (a) shows the orthogonal building layout where each building is rotated 45° to the inundating wave. The channelling of flow still exists, but the flow becomes constricted quicker as the building sizes increase. This layout can only achieve a building density of 50% at 45° before the flow gets pinched. Figure 2 (b) shows the offset version of the orthogonal layout. Currently trouble is encountered when trying to run this version due to a combination of boundary and mesh issues. This particular layout combine both the larger momentum changes on the flow in addition to the pinching of the flow as the building sizes increase and again will pinch the flow at a building density of 50%.

(a)						(b)

Figure 1 (a). Displays the straight building layout. (b) Displays the straight offset building layout.

Figure 2 (a). Displays the orthogonal building layout. (b) shows the orthogonal offset layout.

Results:

The following results are the best fit friction values calculated for the various building scenarios. Figure 3 show the graphical results of table 1. It can be seen in the graph that the straight offset and rotated buildings share quite similar friction values relative to their footprint ratios (and/or building widths). However it is unclear why they are so similar when they were though to affect the flow in different ways. It is not immediately obvious how the restriction of flow behaves similarly to the momentum changes imposed by the offset buildings. But, it does seem logical that both of these building layouts have higher friction values than the straight building layout.

The straight offset and straight buildings also display a levelling off or reduction in friction values even with an increase in building size. This behaviour is not intuitively expected.

| Straight buildings |  |  |  | Straight offset |  |  |  | Buildings Rotated(45) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| width | Footprint Ratio | n | 2nd norm | width | Footprint Ratio | n | 2nd norm | width | Footprint Ratio | n | 2nd norm |
| 3 | 0.0144 | 0.175 | 0.045 | 3 | 0.0144 | 0.175 | 0.047 | 3 | 0.0144 | 0.175 | 0.044 |
| 5 | 0.04 | 0.3 | 0.071 | 5 | 0.04 | 0.3 | 0.073 | 5 | 0.04 | 0.275 | 0.068 |
| 7 | 0.0784 | 0.55 | 0.112 | 7 | 0.0784 | 0.55 | 0.111 | 7 | 0.0784 | 0.55 | 0.101 |
| 9 | 0.1296 | 9 | 0.096 | 9 | 0.1296 | 9 | 0.098 | 9 | 0.1296 | 7 | 0.112 |
| 11 | 0.1936 | 12 | 0.087 | 11 | 0.1936 | 14 | 0.047 | 11 | 0.1936 | 12 | 0.080 |
| 13 | 0.2704 | 16 | 0.070 | 13 | 0.2704 | 19 | 0.030 | 13 | 0.2704 | 19 | 0.050 |
| 15 | 0.36 | 20 | 0.059 | 15 | 0.36 | 33 | 0.014 | 15 | 0.36 | 31 | 0.017 |
| 17 | 0.4624 | 25 | 0.049 | 17 | 0.4624 | 53 | 0.014 | 17 | 0.4624 | 56 | 0.035 |
| 19 | 0.5776 | 27 | 0.045 | 19 | 0.5776 | 50 | 0.028 |  |  |  |  |
| 21 | 0.7056 | 27 | 0.048 | 21 | 0.7056 | 70 | 0.037 |  |  |  |  |
| 23 | 0.8464 | 31 | 0.047 | 23 | 0.8464 | 99 | 0.045 |  |  |  |  |

Figure 3. Plot of friction behaviour of the various building layouts with respect to building size.

Table 1. Tabulated values of the best fit friction values produced by CCS.py.

Discussion:

The results calculated using the python script CCS.py, infer behaviours that are both intuitive and counterintuitive. It does seem obvious that the straight buildings provide the least amount of resistance to the inundating wave, as the quasi-channels are preserved for the inundating wave right up until pinching at a building density of 100%. At the pinching point the water is effectively stopped from progressing through the buildings, with water levels being zero past the pinch. This scenario is difficult to model using friction values, as even very high friction values (10 000+) cannot represent this effect. Secondary considerations that even though a particular friction value might be the best fit, it may not be a good fit as compared to other building scenarios. The second normal result is a measure of this and can indicate the goodness of fit for the best fit friction value. Figure 4 shows the second norm results for a large range of Manning’s n values. For large friction values (100+) it can be seen that the friction water levels do not change much as their second normal result does not change much relative to the building scenario they are being compared too. For larger building widths (high building densities) their becomes no obvious dipping point were there is an obvious best fit. Some of these second normal results tend asymptotically towards a particular value with only a tiny change upwards to identify a best fit result. Some of these friction values are good fits with low 2nd norm results, but it must be noted that the limitations of system must be understood before complete faith can be placed in the suitability of a friction value.