Report structure:

- Results Summary + brief methodology

- Introduction

  - Momentum sinks

  - SWWE’s

  - ANUGA, work produced solely in ANUGA > limitations on scope

  - Manning’s n

- Methodology

  - Introduction

  - Open channel flow similarities/ Domain

  - Building scenarios

  - Friction scenarios/ values

  - Data interrogation

    - CCS.py (Rel. RMS) & PCS.py

- Experiments

  - Detailed analyses

    - All buildings scenarios, 20m wave @ 1000s

    - CART results

  - Wave height investigation

    - All wave height, all scenarios @ 1000s

    - CART results

  - Time dependency

    - Straight buildings @ 100s, 200s, 300s, 500s & 1000s,    for all wave heights

    - CART results

  - Datum error analyses:

    - Reasons for ignoring time and wave heights and using a datum

    - Table of results

- Discussion

  - Best fit friction results

  - Similarities between building scenario friction results why/why not?

  - Building variations effects why/why not?

  - Hydrodynamic behaviour of the model and its implementation and limitations friction should not be taken as actual value, only used as a measurement/ quantification tool.

- Conclusions

  - Shape and length comparisons haven’t been performed

- Acknowledgements

8.	Introduction:	4

Introduction:	4

Momentum Sinks:	4

The ANUGA software:	5

Current methods:	5

9.	Methodology	7

Introduction:	7

Hydrodynamic behaviour:	9

Building scenarios: considerations and limitations	9

Building layouts and parameters:	10

Data interrogation:	12

10.	Experiments:	15

3.1	Detailed analysis:	15

Introduction and Methodology:	15

Results and discussion:	15

CART analysis:	19

Conclusions:	19

Wave height investigation: All wave heights and scenarios @ 1000s	21

Introduction and methodology	21

Results and discussion:	21

Time dependency	25

Introduction and methodology:	25

Results: Time dependant behaviour of friction values	25

Conclusion:	28

Errors of errors:	29

Introduction and methodology:	29

Results and discussion: wave height comparison	29

Results and discussion: time datums comparison	32

Conclusions:	35

Discussion (limitations):	37

11.	Conclusions:	39

Further work:	39

# Introduction:

### Introduction:

In recent years the accuracy of inundation modelling has bee desired to be improved. Currently most inundation models only take into account topography and bathymetry, the effect of surface friction and small scale obstructions such as building have been neglected.

Modelling of tsunami inundation using the SWWE’s have been very successful, however these models are only based on bathymetry and topography in addition to the hydrodynamic behaviour of the fluid. The resistive capabilities of urbanised areas to provide a momentum sink (velocity slowing, mass reducing) has not been quantified in a suitable form to be employed in the current model. It is supposed that the momentum sink effects of a modelled urban environment can be matched to the momentum sink effects of a modelled friction environment. The SWWE’s include a forcing term that can take into account the topographic bedslope and the friction bedslope. The friction bedslope is an equivalent measure of bedslope induced by the frictional resistance or surface roughness between solid objects and the fluid. By using the ANUGA code a Manning’s friction coefficient was calculated by finding a best fit friction coefficient scenario to the given building scenario. The matching was performed by using the relative RMS test, which compared each friction scenario to the given building scenario. The matched friction values can both be used to represent a similar momentum sink in an inundation scenario and as a measurement tool to investigate building parameters of an urban environment.

### Momentum Sinks:

Momentum is a measure of the inertia energy of a moving object (p=mv), the momentum of an object can be reduced in two ways, either reduce its velocity (which is the common method) or reduce its mass. For a fluid, the mass (proportional to volume) is readily changed both spatially and temporally. It is supposed that the friction forcing terms can be used in the SSWE’s to reduce the velocities of the flow bringing about a momentum sink. The size of various friction values will be explored to get a range of momentum sinks that can be used to represent a momentum sink effect caused by an urban environment.

### The ANUGA software:

ANUGA is a finite-volume method for solving the shallow water wave equations for modelling dam break simulations (Zoppou and Roberts, 1999). It has also proved highly capable of modelling the inundation of tsunamis and storm surges (Nielsen, O., et al 2005). The inputs into the code include the boundary conditions (i.e. dam break, flooding or tsunami) in addition to the forcing terms which include the bedslope (generated from topography and bathymetry) and friction coefficients. These are grouped into the source term, which provides for external forces acting on the water volume. The forcing terms have up until this point been restricted to the topography/bathymetry and a blanket ‘urban’ friction value applied to the entire topography surface. Variations in building densities that exist in the inundation path are thought to provide additional resistance to the blanket urbanised friction value currently employed in the model. Methods have been developed for matching a suitable Manning’s friction coefficient (n) for a given building scenario that estimates the additional resistance encountered by the inundating wave with moderate success.

### Current methods:

The forcing terms incorporated into the SSWE’s have currently not been exploited to represent the changing frictional resistance that occurs over a highly variable coastal environment. Currently a friction value (Manning’s n) estimated through empirical studies is assumed to represent the entire urbanised environment. The use of such a strategy can lead to inaccurate water depths (w.r.t) over large scale coastal inundations.

Manning’s coefficient n was created to assist designers of hydraulic pipes and open channels to better predict the frictional effects of various materials on the hydrodynamic behaviour of the fluid. The coefficients for this formula have to be empirically derived from fluid mechanics flume tests. Some work has been developed to correlate the relative roughness (protrusion into the flow) with Manning’s n; however this has only been performed for smaller scale pipe flows for civil engineering purposes. The relative roughness is a normalised measure of the protrusions over a unit distance, with the protrusions typically measuring less than 1cm, this protrusion length is generally much smaller then the depth of water over the protrusion. It is unsuitable for this application as the ‘roughness’ (simulated buildings) protrudes through the entire water column which will act on the fluid in a different way. Subsequently equivalent friction values are to be developed empirically using the ANUGA code. This will be performed by comparing simulated building scenarios against artificially created friction scenarios. Since the friction components of the SSWE’s using Manning’s coefficients derive from open channel flow/fluid mechanics, methods that are used to derive Manning’s n will be used for the comparison of the building and friction scenarios.

# Methodology

### Introduction:

The aim of this report is to identify friction values for building scenarios that will represent the best and worst cases in terms of a momentum sink to the inundating tsunami. The building scenarios will be created for a range of building widths in various the layout patterns and orientations. This is required because the hydrodynamic behaviour of each building scenario is anticipated to change as the building dimensions change. The chosen layouts and orientations of the building scenarios should provide the limiting cases (best and worst) for the friction values that could be applied to a real world urban environment.

Fluid mechanics similarities (open channel flow):

Manning’s n history in open channel flow made it a logical choice to constrain all the inundation scenarios to the open channel flow criteria. Manning’s n friction coefficient was primarily derived for use in open channel flow problems so that engineers could predict the fluids hydrodynamic behaviour. Open channels are either natural rivers/streams/creeks or manmade channels relying on gravity for the movement of water. The domain of the inundation scenarios will be constrained to a rectangular open channel flow problem, where water will inundate from one side through the building scenario and then out the other side.

Open channel flow terms shall be used to describe the hydrodynamic behaviour of the scenarios. Open channel flow problems can readily be solved once they reach a steady state. The steady state term refers to the stabilisation of the input and output flows and the velocities and water heights in the problem domain. The fluid reaches equilibrium between flows in and out of the system and the gravitational and friction forces acting on the fluid. From initial prototyping, the steady state point is also the only time that hydrodynamic behaviour can be compared between scenarios, as only smaller stage heights occur up to the steady state point. There are hydrodynamic differences between the frictional and building inundations up till the steady state point. Generally the friction scenarios will take longer to reach the steady state than the building scenarios, but the hydrodynamic behaviour of the friction scenarios before the steady state point is dependant on many other variables. Current building damage models only include the water depth as an indicator for building damage. Based on this fact, the only variable that will be matched between the scenarios will be the stage heights (at the same time interval) in both the friction and building scenarios. At later stages when momentum affects have been studied for buildings this information can be extracted and possibly matched using similar methods that have been used for this project.

The following domain settings have been chosen to best replicate an open channel flow problem. They should promote flow in only a single direction (or 1D) to remove any ambiguity arising from complex velocities within the scenario. The wave is able to inundate from the left hand side, progress through the building or friction zone then proceed out the right hand end of the scenario. The sides of the scenario are frictionless boundaries, which should give a constant cross section along the entire channel. The frictionless sides are the only difference to a typical open channel flow problem and should allow the width of the domain to represent a unit width. A blanket friction value of 0.01 (smooth concrete) has been applied to the other areas in the domain as this would represent that absolute minimum friction coefficient likely to be encountered by the inundating wave.

200m

breadth/

depth

200m

WAVE

zone

Depth/length = 1000m

This domain shall be used for all of the building and friction runs. The large distance behind the modelled area is to remove boundary reflections or water build-ups that will interfere on the final max water level in the zone modelled by ANUGA. The small distance at the start of the scenario is to ensure the maximum wave height occurs at the start of the zone.

### Hydrodynamic behaviour:

It is unknown what determines the resistive capabilities of an urban environment to the inundating tsunami wave. There is also a lack of a measurement system that can quantify the resistive abilities of an urban environment. This system is needed to convert the parameters of the urban environment to a similar modelling building environment that has similar hydrodynamic characteristics. Building width (relative to block size) seems to be an obvious candidate, as a larger building width reduces the amount of flow that can easily pass through the scenario. The offset of the buildings is also believed to be an important issue, as it prevents straight line travel and forces more redirection of the water as it passes through the scenario. In Fluid mechanics, every time a fluid has to change direction it imparts momentum energy on the object that is redirecting the flow. This imparted energy is taken from the flow, which reduces the net velocities throughout the fluid. Momentum will be lost in both straight and offset scenarios, both rotated and not, as the fluid will be redirected to varying degrees for all the scenarios. Simple variations of the buildings were done to determine what changes would implement higher resistance to the flow.

The only way to quantify the resistance of the building scenario is the compare it to a friction affected zone. The friction needs to be implemented in a the same domain over the same zone as the simulated buildings, so that only the fluid characteristic in the zone are being modelled.

### Building scenarios: considerations and limitations

The action of a fluid passing through a collection of obstacles is a common idea in wind engineering and in some flooding studies. However case studies of open channel flow through a collection of vertical obstacles is difficult to find in the literature. Similar studies have been performed for forested flood plains by the USGS and other organisations. This type of work however has not characterised the forest parameters into anything readily transferable to an urban scenario. The likely ‘building densities or porosity’ of the forested areas is small in addition to being combined with vegetation effects. These derived Manning’s n values are still small when being compared to the potential Manning’s n sizes for this project.

Various building scenarios were created to investigate the changes in building orientations would impact the flow of water around them. The ratio of open space available to the inundating water is thought to be a major contributor in restricting the advancing water. Other parameters, such as the amount of redirection the water undergoes is also thought to contribute to the reduction of energy in a flow. The rotation of individual buildings, decreases the amount of open space but may also decrease the amount of redirection imparted to the flow.

The blocks and houses have been restricted to squares to simplify the amount of variables that contribute to the resistive capabilities. The introduction of rectangular on larger sided polygon can be investigated later once the fundamental parameters have been established. The size of the block has been taken from the ABS Feature Article - Are we building on smaller blocks? Shows average block sizes and floor sizes for Australia. The block size of 625m2 shall be chosen because of its nice root size of 25m. This size is a little smaller than the average block size for Australia but it should be justified due to the higher land prices near the coast which should produce slightly smaller block sizes. The block size is anticipated not to have an effect on the inundation for large block sizes as the flow is fully turbulent. The building density i.e. house size relative to block size is believed to be more important. The house sizes, thus building densities will be varied from 1.5 - 85% which should provide a good indication of how the building density affects the resistive capabilities. The apparent density to the incoming wave will also be measured to gauge its influence.

### Building layouts and parameters:

Several building layouts have been chosen to represent both the least and most restrictive layout patterns for buildings in an urban environment. All these building layouts have been based on square buildings and square blocks to remove directional preferences for the flow. The chosen layouts are shown below with a brief description of their anticipated behaviour. Figure 1 (a) shows the straight building or high order (i.e. not random). This layout is believed to provide the least resistance of all the chosen building layouts to the inundating wave. There is a clear path or quasi-channel for water to flow through, which pinch or cut off flow once the buildings have reached full density. Figure 1 (b) shows an offset of every second row of buildings of half the block length. This places a building in the path of the water channelling between two buildings. This makes the flow continually change direction which causes momentum changes and thus energy reductions in the flow. This perfect offset is believed to impart the most momentum changes on the inundating flow. For both figures (a) and (b) the building density can achieve 100% until the buildings will touch each other. Figure 2 (a) shows the rotated building layout where each building is rotated 45° to the inundating wave. The channelling of flow still exists, but the flow becomes constricted quicker as the building sizes increase. This layout can only achieve a building density of 50% at 45° before the flow gets pinched (blocked). Figure 2 (b) shows the offset version of the rotated (45°) layout. This particular layout combines both the larger momentum changes on the flow as well as the pinching of the flow as the building sizes increase and again will pinch the flow at a building density of 50%. Another building scenario with half an offset (1/4 of block width) will be used to investigate the importance of the offset in the resistive capability of a scenario.

(a)						(b)

Figure 1 (a). Displays the straight building layout. (b) Displays the straight offset building layout.

Figure 2 (a). Displays the rotated (45°) building layout. (b) shows the rotated (45°) offset layout.

All these scenarios will be created using the ANUGA code. The domain will be consistent for both the friction and building scenarios, the buildings have been adjusted for changes in the building widths to allow the last row of buildings to coincide with the friction boundary. The front boundary will be in different places between the friction and building scenarios, but height differences between these points is negligible as the stage levels nearly always approach the wave height.

The loop_buildings.py and loop_friction.py scripts were used to create the building and friction scenarios respectively. Friction scenarios have been produced for friction values from 0 to 1 in steps of 0.025 and from 1 to 120 in steps of 1. The buildings scenarios have been created for widths between 3 – 23m in 0.2m and 1m intervals depending on the experiment being performed. These files are created as a .sww file (shallow water wave), which stores water height and x and y momentum for each triangle cell for each time step.

### Data interrogation:

A python script (CCS.py, compare cross sections) was used to compare each of the building scenario variations to the 160 friction scenarios. This script interrogates the .sww file for stage depths at gauge locations for a specific time. The cross section of these stage depths are compared to each other using a relative RMS (EQ. 1). This is done at a chosen time interval for the candidate building scenario and the friction scenarios. Each result of the relative RMS tests for each friction value to a building scenario variation was recorded in a csv file, a summary was also generated of the lowest RMS test result for each building variation. This summary report of best fit friction values is the primary data that is used to make interpretation about the behaviour of the building scenarios.

i=0nzi−zi2zi					(1)

Geoscience Australia’s cyclone cluster was employed to perform all the computational work. The screen shots come (Figure 2.2 and 2.3) from the swollen viewer which was used to ensure the correct generation of the .sww file before they were used in the comparisons. The cross sections of the .sww file at a particular time point could also be used viewed by using the PCS.py (plot cross section) script. This was useful for investigating the changes in stage heights of friction and building scenarios at particular points of interest.

The swollen viewer was also employed to visually inspect scenarios before using them as synthetic data. For the larger runs only small samples of all the scenarios was checked for accuracy. The viewer allowed easy determination of whether the steady state point had been achieved. This was also the only way to check the hydrodynamic behaviour of the inundation. Although there is a lack of data from the viewer it proves to be an important part of the process of interpretation of the scenarios. Figures 2.2 and 2.3 respectively show both a straight building scenario and friction scenario at the steady state. It is easy to see when the stage levels are matched that more flow can run through the friction scenario than the building scenario.

(a)						(b)

Figure 2 (a). Close up of a simulated building scenario at steady state with the simulated buildings making the ‘holes’ in the water. (b) Close up of friction scenario (n=22) at a steady state time. Note there is no reduction in the cross section.

A combination of these methods will be used to perform the experiments investigating the different parameters affecting the selection of friction values for building scenarios. The flexibility of the ANUGA code allows many variations in both the data that is produced and how it is extracted to investigate the different behaviour of the friction and building scenarios.

Conclusion:

# Experiments:

3.1	Detailed analysis: All scenarios, 20m wave @ 1000 seconds

### Introduction and Methodology:

After initial prototyping, an arbitrary 20m wave was chosen to do a detailed investigation on changes in building widths for all the scenarios. The 1000 second time point was chosen as this is the maximum probable wave period. The 20m wave height and 1000s time point allowed both the building and friction scenarios to reach the steady state point. The steady state point means that maximum water heights have been achieved and removes the time dependency from the scenarios. Detailed building width changes from 3 - 23 m in steps of 0.2 m (100 variations for each scenario) for all the scenarios will give many data points that can be used to assess the matching method. This is important to asses the suitability of deriving friction values for the building scenarios. The matched friction values can then also be used to infer changes in the resistive behaviour based on increases or decreases in the matched friction value.

### Results and discussion:

The following results have been generated from the loop_buildings.py and loop_friction.py scripts then compared using the CCS.py script. The reports are then placed into a spreadsheet program to allow suitable interpretation through the use of graphs and plots. Figure 3.1 displays the raw best fit friction values for the 20m wave at 1000s. Plotting the best fit friction values against the building width (proportional to building density) provides a suitable measure of changes in friction values due to increases in width. All the building scenarios have been designed to provide additional resistance to the ‘base’ straight buildings scenario, which can be clearly seen in fig 3.1. All scenarios have a consistent jump between friction values below 1 to 5-7 at small building sizes.

Figure 3.1. Plot of matched friction values versus building width. The straight buildings have the lowest of the matched friction values. Matched friction values for scenarios start to diverge at building widths greater than 13m.

The sudden jump in the friction value is primarily due to an optimisation error and is present down to infinitesimally small changes in the building width. The point at which this jump occurs is also the point where the worst best fit solutions are chosen, that is the best fit friction value at the jump point has the highest Relative RMS result for all the best fit friction values (Figure 3.1). The higher RMS values also indicate that at this transitional point in the building scenarios the friction scenarios are not the best representations of stage levels. If the best fit friction value was determined by a rate of change criteria this problem may disappear. However the friction values that get jumped still have relatively low RMS errors, which still provide OK matches of the stage levels. The friction values being compared produce some ‘noise’ or variance due to the similarity of the RMS results of adjacent friction values. This effect becomes more pronounced for higher friction values as changes between consecutive higher friction scenarios become smaller. Figure 3.3 shows the RMS errors of the full range of friction values for a selected range of building scenarios. The optimisation can be seen as obvious minimum points at friction values below 1 and above 5-7. It is hard to infer why this is happening, but at these friction values the stage levels do not match as well with the building scenarios. Whether this is due to a transitional point in the building scenarios (around 5-7m building width) which the rate of change of the friction scenario cannot represent is hard to ascertain.

Figure 3.2. Relative RMS errors plotted against building width. The peaks occur between building widths of 5-7m. The peaks correspond to jumps in the matched friction values.

Figure 3.3. This plot shows the range of RMS errors for friction values for a selected range of straight building scenarios. The minimum points for each curve represent the best fit friction value.

Additional resistance due to the scenario variations are not significant until around 13m, where the differences between the building scenarios begin to change the hydrodynamic behaviour and thus the best fit friction result. Figure 3.1 shows all the building scenarios have similar friction values up to the 12-13m building widths. This would indicate that different orientations and layouts at lower building densities do not affect the resistive capabilities as much as the building density itself does. As anticipated the straight buildings scenario proves to have a base level of friction coefficients.

The rotated and rotated offset buildings have very similar friction results, that indicates the offset is not significant when the buildings are rotated. This might be a result of the reduced flow entering a relatively higher open space volume; this reduces the velocities so when they change directions minimal momentum energy is lost from the fluid. This effect is not observed in the straight buildings (offset or not) as the spaces for the fluid have constant widths throughout the scenario. Because the fluid is travelling faster when it changes directions the momentum changes impart a greater loss of energy onto the flow. However it is hard to separate the individual contribution of flow restrictions and flow redirection

It can be seen in the graph that the straight offset and rotated buildings share quite similar friction values relative to their footprint ratios (and/or building widths). However it is unclear why they are so similar when they were though to affect the flow in different ways. It is not immediately obvious how the restriction of flow behaves similarly to the momentum changes imposed by the offset buildings. But, it does seem logical that both of these building layouts have higher friction values than the straight building layout.

The straight offset and straight buildings also display a levelling off or reduction in friction values even with an increase in building size. This behaviour is not intuitively expected as the smaller gaps should reduce start to increase the flow constriction and at this stage it is not understood. However the friction value is not a direct measure of the buildings resistance, only the stage levels produced by the friction value is compared to the buildings stage levels. This idea limits what can be directly inferred by the matched friction value.

### CART analysis:

The 20m wave best fit friction values were put into the regression tree analysis software CART, this software generates a classification/regression tree to predict a target variable based on a number of predictor variables. CART is very powerful software for classifying very complex data and can generate rules to be applied on future data sets. In this instance CART was used infer at what stage the predictor variables had an influence on the best fit friction value. The best fit friction values of all the scenarios was used with the width, rotation and offset as the predictor variables and Manning’s n as the target variable. The optimal tree chosen by CART produced too many splitting nodes to be used effectively as a general classification of the behaviour of the friction scenarios. While the Error was low (about 0.07) a smaller tree was chosen with less nodes but a slightly higher error (= 0.098). A smaller tree with a less than 10% error is deemed suitable for inferring friction values from the dependant variables. See appendix A for a graphical representation of the graphical tree.

For building widths less than 14m the width is the only factor for a friction value. This seems logical as smaller building widths may not begin pinch or obstruct the flow at lower building width (densities). For the larger building widths the complexities of each scenario begin to emerge. The rotation and offset begin to impart more significant flow restriction and velocity reduction, such as when the rotation starts to increase the apparent width increases and pinches the water flow. The amount of offset seems also contributes to a higher friction value, however differences between the half offset and full offset scenarios are not consistent and this behaviour is reflected in the tree. Unfortunately the CART analysis can only confirm the behaviour of scenarios do the changes in the predictor/determining variable it cannot suppose reasons for behaviour that is counter intuitive.

### Conclusions:

Changes in the resistive abilities of an urban environment have been investigated using various layouts and orientations of simulated buildings. The range of orientations and layouts of the scenarios has aimed to bracket real world friction values through using simple to extreme scenarios. As anticipated the straight buildings scenario has proved to be the least restive of all the scenarios. However the parameters such as rotation and offset don’t become significant until around a 25% building density has been achieved. After this density the rotation and offset contribute in complex ways to produce extra resistance. It is also difficult to measure similar offsets and rotations in a real urban environment. For now, only building density should be used as this will produce better but slightly conservative results, which are more appropriate, given some of the limitations with using a friction value. Further tsunami variable such as the wave height and inundation time will be investigated to assess what impact they have on the matched friction value.

## Wave height investigation: All wave heights and scenarios @ 1000s

### Introduction and methodology

The Manning’s n implementation in the SWWE’s indicated that the friction bedslope was dependant on the depth of water (specifically the hydraulic radius) in addition to a specified Manning’s n value. If the matched friction value is wave height dependant, it makes the implementation of the friction value in the ANUGA code difficult because for a given cell ANUGA cannot predict a the maximum wave height. All the scenarios were investigated over wave heights 2m, 5m, 10m and 20m, using a smaller building width density. Loop_buildings.py was used to create the extra 2m, 5m and 10m building scenarios from 2-23m widths in 1m steps. The larger changes in the building widths was done to speed up to computational time of both the generation of the .sww files and of the comparison scripts which 24hrs plus to work.

### Results and discussion:

The following results have been extracted from reports generated from the CCS.py script interrogating all building scenarios at the 1000 second time interval. The plots are wave comparisons of the change in matched friction value through a change in wave height. Figures 3.4, 3.5, 3.6, 3.7 and 3.8 all show a separate building scenario with a comparison over wave height. For all the scenarios the lower the wave height the lower the matched friction value, this is occurs because the smaller wave height (lower water depth) generates a bigger friction bedslope to slow down the water. Although the 2m, 5m and 10m waves have a smaller resolution, the characteristic behaviour of the detailed 20m wave gets preserved in the smaller waves. However the kinks and changes in the lower wave matched friction values do not give any better insights into why these phenomena occur.

The difference between the 2m and 20m wave matched friction values is about 10 fold. This difference is possibly large enough to cause even larger errors in the stage height when the wrong wave height estimations are made. It is difficult to estimate what the wave heights when topography effects are taken into account. The solution might have to be iterative involving many inundations run with modifications of the friction value to take into account likely maximum wave depths at the point.

Figure 3.4. Straight building scenario, best fit friction values vs building width.

Figure 3.5. Straight half Offset building scenario, best fit friction values vs building width.

Figure 3.6. Straight Offset building scenario, best fit friction values vs building width.

Figure 3.7. Rotated 45° building scenario, best fit friction values vs building width.

Figure 3.8. Rotated 45° Offset building scenario, best fit friction values vs building width.

Conclusion:

It has been shown that the matched friction value is heavily dependant on the wave height. The errors of using a matched friction value at a particular wave height when another wave height inundates will have to be investigated to avoid inaccurate final max water stages. The use of a datum wave will have to be explored to assess what range of error will occurs when differences in wave height or time occur.

## Time dependency

### Introduction and methodology:

From the wave height tests it was seen the inundating behaviour of the friction scenarios is dependant on the wave height. The friction scenarios were slower to inundate for smaller waves than the building scenarios, which suggests time based wave behaviour for the different wave heights (for the friction scenarios). Different times will be investigated to judge whether steady states have been achieved; otherwise time will become another variable for the selection of a friction value for a given building scenario.

### Results: Time dependant behaviour of friction values

The straight buildings scenario was used to investigate the time dependency of the friction matching process. From the previous wave height investigation, it was noticed that both friction and building scenarios did not develop fully to steady state for smaller wave heights. This becomes important as the period for most of the damaging tsunami waves is between 100 - 1000 seconds. Comparisons were performed for the straight building scenarios for all wave heights at 100s, 200s, 300s, 500s and 1000s. Friction scenarios were compared at the same time points, to ensure consistency when friction values are employed in the modelling. Matched friction scenarios for this project were nearly always slower to inundate and reach steady state than the building scenarios, irrespective of wave height. The wave height dependency of the friction scenarios slowed down the inundation progress of the wave front especially for the smaller waves. This slowing effect (for friction scenarios) for the smaller waves produced smaller best fit friction values for a candidate building scenario. This is due to the smaller friction values producing larger stage heights that were closer to the candidate building scenario. Figures 3.9, 3.10, 3.11 and 3.12 show the 2m, 5m, 10m and 20m wave height best fit friction values for a range of time intervals. The increase in wave height across these figures shows the increase in friction values. The range of matched friction values between the 100 seconds to 1000 seconds time interval is at least 10 fold for the majority for all but the lowest building widths. This is due to the lowest building widths providing minimal resistance, which allows a greater inundation for smaller time intervals.

Figure 3.9. Straight building scenario with best fit friction values at specific time intervals.

Figure 3.10. Straight building scenario with best fit friction values at specific time intervals.

Figure 3.11. Straight building scenario with best fit friction values at specific time intervals.

Figure 3.12. Straight building scenario with best fit friction values at specific time intervals.

Smaller friction values get matched at smaller time intervals because the smaller friction values allow greater inundation through the scenario. The building scenarios generally inundate faster than the friction scenarios, which means smaller friction values get matched to keep up with the higher stage levels.

### Conclusion:

The time dependency of the matched friction values can only be removed when both scenarios reach a steady state, which is also the same time when the maximum stage heights are achieved. Unfortunately the steady state does not occur for the time intervals we are looking at.

The friction matching at various time intervals has showed that time is a very important variable for the matching the friction values. This is due to the different inundating behaviour of the building and friction scenarios. Since a time based friction value cannot be effectively implemented in the ANUGA code, an analysis of the time associated errors will be made to gauge the possible uncertainties in stage levels over time intervals between 100 – 1000 seconds.

Errors of errors: How much does friction matter?

### Introduction and methodology:

The process of matching a friction value for a given time and wave height is relatively easy. This is easy when using scenarios and domain where Manning’s friction coefficients were originally designed for. Unfortunately the implementation of the friction in an inundation scenario is much more difficult because the time and water height dependency of the friction bedslope equation. The water heights and inundation times for individual cells need to be predicted before they can be calculated to reduce stage height errors. However the implementation of time or water depth dependant friction values in the ANUGA code cannot be achieved to reduce these errors. The friction values have to be set like the topography and bathymetry as a boundary condition. The errors of best fit friction values of a certain wave height in another wave height will be calculated, this will also be done for time as well. This process involves choosing a datum set of best fit friction values and them using them at other time or water heights, the errors of this process were then used to assess the suitability of fudging friction values to remove the time and depth dependencies.

The errors of friction values set at a 5m datum will be used the find the RMS errors of using these friction values against building scenarios of different wave heights and time intervals.

### Results and discussion: wave height comparison

Figure 3.7 shows the RMS results of the best fit friction values for the straight buildings with a 5m wave. The errors of the 5m best fit friction values for the 2m, 10m and 20m wave for the 1000s time interval are shown. Figures 3.13, 3.14, 3.15, 3.16 and 3.17 show the 100s, 200s, 300s, 500s and 1000s intervals for the straight buildings 5m wave scenario. The friction value datum’s are the 5m wave at the time interval being used at a different wave height. These figures show the RMS errors of using those friction values in another wave height. The 2m wave obviously has the highest wave error for all the time intervals for all but the smaller building widths.

Figure 3.13. Wave comparison with a 5m datum wave at 100 seconds.

Figure 3.14. Wave comparison with a 5m datum wave at 200 seconds.

Figure 3.15. Wave comparison with a 5m datum wave at 300 seconds.

The very large errors of the 2m wave would be associated with the very slow inundation of a 2m wave friction scenario. This causes best fit friction values that are much smaller than the 5m, 10m and 20m waves. So when the 5m friction values are used in a 2m scenario a large error is calculated. The large error of the 2m wave gets progressively smaller as the time interval increases because it has inundated further and begins to ‘catch up’ with the other scenarios. The wave height dependency is due to the smaller water depths decreasing the inundation speed. If enough time was given to achieve steady state then the errors between different wave heights should become very similar. This effect can be seen in figure 3.17 were the errors are all below 0.15, the 10m and 20m waves have very similar errors as their inundation speeds are closer. For all these time intervals the very small building width give the smallest RMS errors even when using other best fit friction values.

Figure 3.16. Wave comparison with a 5m datum wave at 500 seconds.

Figure 3.17. Wave comparison with a 5m datum wave at 1000 seconds.

### Results and discussion: time datums comparison

A time based comparison using best fit friction values at a certain time were used to determine what errors the friction values would generate when used at other time intervals. This has only been performed on the straight buildings scenario with a 5m wave for different time interval datums. Figure 3.18, 3.19, 3.20, 3.21 and 3.22 display the RMS errors of this time datum comparison. The progression through the intervals so that overall friction values chosen at 100 sec provide the best level of RMS errors when used at longer time intervals. Figure 3.18 shows that the errors of the 100s friction values are less than 0.2 and grouped together well. As time progresses the smaller wave height start to generate large errors, which is due again to the larger friction values significantly slowing down the inundation. It should be pointed out that on some graphs the datum errors even though they are best fit have greater error than some other time interval that are not using their best fit friction values. This is because some time intervals have on the whole lower RMS values the other variation or scenarios. Again very small building width have the smallest errors.

As the time interval increases, the errors of smaller time intervals increases significantly, up to 0.55 for a 5m wave at 100s using 1000s friction values. It can easily be deduced that the best strategy to help remove time related errors is to use smaller rather than larger friction values. However the time related errors are still large and would have a combination effect with wave height errors.

Figure 3.18. Time comparison with a 5m wave at 100 seconds datum.

Figure 3.19. Time comparison with a 5m wave at 200 seconds datum.

Figure 3.19. Time comparison with a 5m wave at 300 seconds datum.

Figure 3.20. Time comparison with a 5m wave at 500 seconds datum.

Figure 3.21. Time comparison with a 5m wave at 1000 seconds datum.

### Conclusions:

The error of errors due to time and wave height indicates that there are certain situations that are better than others. To help remove time errors, picking friction values matched at a smaller time interval is better than a longer time interval. However the would also be a lower time limit to this benefit. A time interval analysis of the wave heights showed that as the inundation time increase the wave height dependency of the friction values becomes less when using a 5m datum wave. The use of friction values from a 2m wave may reduce the overall error against a higher wave friction values and improve the consistency of the error. However these errors are still relatively large, especially for moderate to large building widths.

It is difficult to estimate what are good datum values to use for an individual problem as changes will produce a different range of overall RMS errors. It seems from this analysis that you get smaller errors when you under estimate the inundation time and water depths for a given matched friction value. Even though certain strategies can reduce errors between wave heights or time intervals the overall RMS error of the stage heights start to get bigger. These errors may approach 30 - 50 % even with strategies to limit the time and wave height dependency. Decisions should be made on a case by case bases both before and after a simulation, if errors of this size are acceptable outputs compared to the situation of no friction values.

# Discussion (limitations):

The results calculated using the python script CCS.py, infer behaviours that are both intuitive and counterintuitive. It does seem obvious that the straight buildings provide the least amount of resistance to the inundating wave, as the quasi-channels are preserved for the inundating wave right up until pinching at a building density of 100%. At the pinching point the water is effectively stopped from progressing through the buildings, with water levels being zero past the pinch. This scenario is difficult to model using friction values, as even very high friction values (10 000+) cannot represent this effect.

Internal shear forces in the fluid are not taken into account through the SSWE’s. Fluid phenomena such as vortex shedding generated from separation point simply cannot be modelled by the ANUGA code. These effects are important to realise if some form of verification is done in a flume because these fluid phenomena will most certainly be present.

One of the problems with using the friction values as a form of measurement systems is that you expect it to be consistent linearly like a ruler or log based like decibels. The problem with this system is that none of this is guaranteed the consistency of changes in friction values is not known enough to make absolute judgments about what is happening to the flow.

# Conclusions:

The results show that friction values can be successfully matched to building scenarios using the second normal test to find a best fit friction value for a particular building scenario. The goodness of this fit will need to be considered in relation to the accuracy of the friction value replacement as although the friction value may be the closest representation of the maximum water levels. The hydrodynamic behaviours of the two scenarios may be significantly different. However for initial purposes the method of comparing friction and building scenarios has been established. The integrity of this method is only a result of the accuracy used in comparing the two results. Variations in the current comparison method will lead to minor changes in the matched friction values, however this should not lead to significant changes in its implementation in the inundation model as the water level will only change by small amounts.

### Further work:

The author believes that flow restriction through a modification or scaling factor applied to the fluxes between triangles will better replicate the hydrodynamic behaviour of the building scenarios. In most inundation scenarios for the Australian coast the buildings (provided they stand up) will protrude through the inundating flow. This provides a constant restriction throughout the water column, which logically holds back water. This is fundamentally different from frictional resistance at the base of the flow which relies on the viscous forces of the fluid reducing the velocities. This occurs when the higher velocities of the shallower water are slowed against the slower velocities near the bed slope with friction (boundary layer fluid mechanics).

The flow restrictions would be implemented in the domain function but would apply to the fluxes between elements as opposed to the forcing terms. This could be implemented as a porosity or permeability value applied across the applicable zones. Issues may be encountered with the sizing of the triangle cells, as larger meshes with smaller triangles with the same flow resistance value may reduce flow more severely then smaller meshes with larger triangles. (However it may also not be an issue because less flow will be travelling through a smaller triangle).

Friction coefficients (n < 1) will still be appropriate to be applied to the ground surface between buildings, as paved, grassed and planted areas will still provide friction resistance at the base of the flow. The areas in an urban environment are finite in nature and can easily be quantified by existing frictional values calculated by hydraulic engineers (in tables by USGS for example). The finite nature of the restriction that they impart on the flow makes frictional assumption valid.

The changing size in the inundating wave should also have an effect on the friction scenarios. The forcing terms that determine the bed friction on the wave are dependant on water depth (hydraulic radius), velocity and Manning’s n. The friction bed slope will increase as the wave depths inundation depth decrease. This depth dependant behaviour of the inundating wave over a friction surface is different to the flow restricted inundation through a buildings scenario. This makes the comparison between the two scenarios difficult because scaling effects of the wave height in the friction scenarios will behave different to the building scenarios.

The use of friction values at higher densities or low porosities does not replicate the hydrodynamic behaviour of the inundation. However for a large majority of the coastline most areas are still suitable to be modelled by a friction value, such as grass, pavements, sand and water. It is only in moderate densities such as housing where the use of friction value can become unsuitable. The main issue extends from the water depth (hydraulic radius) dependency of the Manning’s friction formula relative to a moderately dense building scenario preventing (stopping) a percentage of the water travelling through the scenario. To be able to model this behaviour a porosity forcing term should be included into the SSWE’s. This porosity term would affect the flux terms between the cells and would be a pure restriction on the fluid allowed to travel between individual cell walls.