Work Plan:

Introduction:

Define a set of criteria to analyse the success of the Urban friction coefficients. Initial parameters will be identified that are believed to impact on the Frictional resistance of an urban zone.

Aims and outputs:

The final output of this project is to provide specific Manning’s friction value that imitates specific built environment scenarios. i.e. suburban, commercial and CBD building environments. In the ANAGU model friction values are much easier to apply and much less computationally expensive than modelling the actual buildings. This becomes important for large inundation scenarios as it will increase the accuracy of the modelled inundation, while being less resource intensive.

Methodology:

Computational simulations will be performed on two base scenarios. The first scenario will model the inundation effects on buildings representative of urban environments in the ‘real world’. The second scenario will model the effects of an arbitrary Manning’s friction value that ultimately will be used to replace the building models that greatly increase computation time (and or resources).

Primary outputs:

- Investigate hydrodynamic behaviour of both simulations to determine the suitability/limitations of friction value replacement for urban areas.

- First match Manning’s friction values to highly ordered building scenarios.

- Investigate the changes to apparent friction as building order decreases.

- Produce a series of charts/equations that empirically prescribe a manning friction value for given urban parameters. Or modification parameters (multiplicative or addition factors) for increasing disorder in components of urban design/layout could be used to alter friction values for high order situations.

Potential parameters:

- Urban density Breadth

- Urban density Depth

- Building order, both randomness and rotational placements.

  - To be investigated after high order building placements.

- Size effects, Suburban vs. CBD; is building density the only important criteria?

- Edge effects need to be considered from boundaries.

  - i.e. select suitable region away from edge effects and rear boundary.

- Inundation wave Depth

Tsunami Basics:

The purpose of this comparison is to minimise the influence of the many contributing factors to a tsunami inundation. The modelled surface will be flat to negate topography effects. The inundation wave will be analysed in cross sections common the both models. The maximum water depths will be compared for a range of Manning’s friction values to best represent the building scenario. The modelled depths will be 5, 10, 20, 30m.

Inundating Hydrodynamic behaviour and limitations:

The main modelled outputs of the ANAGU software consists of water depths and momentum. Both of these properties are important for the damage models based on the inundation. Currently only water depth is used to predict damage to buildings however in the future momentum may also need to be considered for building damage. The comparison of the friction and buildings should try to keep similarity between both of the parameters.

The comparison of these two effects may initially be difficult as the two problems are dependant on two different hydrodynamic behaviours. The building models seem to model more restriction of flow as the wave progresses. As a result the wave front velocity decreases the further into the building zone as the volume of the wave front decreases.

The frictional block that represents the building zone still produces a back up a flow at the zone boundary. However the progression of the wave front is slowed only by the frictional forces, which only occurs at the bed surface. Subsequently flow backs up behind the progressing wave front, as the internal and external shear forces fight against each other.

Friction zone

INCOMING WAVE

The bed slope friction prevents runout of wave base; the wave/water momentum catches up with the slowed wave front. This wave shape does not replicate the wave shape as seen through the modelled buildings. As the wave progresses over the friction ‘block’ its velocity does not decrease. Due to a large enough depth of water immediately behind the wave front to roll over the slow water, thus given a steady speed.(?)

Enough scenarios of both these aspects have not been explored enough to gauge whether buildings can be represented by a singular friction value. Running both simulations for long inundation depths will be required to estimate the ‘steady state’ water surface which will indicate the maximum water depth over the given period. All simulations should be performed over very long periods. This is to provide a steady state water level to measure the maximum achieved depths from.

Broad assumptions have to be made to allow a comparison of friction scenarios to building scenarios. Maximum stage heights throughout the scenarios seem to be the best comparison method, as this is the direct output required for the damage modelling. Other properties such as velocities and flow travelling through the scenario may also have to be compared as these also have notable effect, but are not currently incorporated into the model.

Fluid mechanics similarities:

The inundation scenarios will be controlled in such a way that they will represent a hydrodynamic problem similar to an open channel flow problem. Open channel flow problems are regularly analysed in hydraulic engineering applications and are what the manning friction values are primarily calculated for. The domain of the inundation will be restricted to a rectangular open channel flow problem, where water will inundate from one side through the building scenario and then out the other side. The dynamic wave inundation will be allowed to reach a steady state. This is a fluid mechanics term that states that the properties and geometry of the flowing water have reached a steady state (or equilibrium), even though the water is still flowing through the scenario. The steady state point is where the fluid tries to obtain an equilibrium of the forces acting both externally an internally on it. A fluid will always want to achieve this, and generally will, as long as the input and output forces are steady and given enough time.

Open channel flow terms shall be used to describe the behaviour of the inundated scenarios in the steady state, as this is when the maximum stage heights are achieved. The steady state point is also the only time that hydrodynamic behaviour can be compared between scenarios, as dynamic behaviour only produces smaller stage heights up to the steady state point. Time is a very important factor in what the fluid is doing. The inundations for example may not actually reach steady state during an actual tsunami and then the dynamic behaviour of the inundating wave becomes important. However the dynamic behaviour of the building and friction scenario cannot be dynamically compared as they are two different hydrodynamic behaviours (apples and oranges)

Initial comparisons:

The first comparisons will use arbitrary buildings porosity values of 0.2-0.8. The friction values will then be chosen the best represent these building porosity values in 0.2 value increments. More detail may be needed around the larger porosity values (smaller building density) as small changes in porosity may greatly change the corresponding friction value. The building order will be very high, with all models consisting of a typical grid pattern. Irregularities or disorder (realistic urban scenarios) will be investigated at a later stage once the problem fundamentals are understood.

Final domain settings:

The following domain settings have been decided on as they should remove most size and boundary effects from the scenarios.

200m

200m

1000m

WAVE

This domain shall be used for all of the building and friction runs. The large distance behind the modelled area is to remove and boundary reflections or water build-ups that will interfere on the final max water level modelled by ANAGU. The small distance between the wave start and the buildings/friction is due to the need for a constant wave surface in order to approach a steady state situation.

Comparison methods:

The outputs of both scenarios will be a sww files for long time periods t = 0-1000s. The water depths will be compared for the final (t = 1000s) cross section down the middle of each section. The edge/boundary effect are minimal and the cross-section through the middle will not be affected by any slight changes on behalf of the boundaries. The two cross sections are essentially signals which have to be statistically compared to each other on a best fit basis. The RMS (2nd normal) test will be used as it is a measure of the standard deviation or the differences between the signals. This is easily implemented in python and automation can be performed to find the best fit friction value based on this test. However the suitability of the best fit water level may not be suitable as the water passing through volumes and velocities, may not match the building scenario that they are trying to replace.

Experiment 1:

Aim:

Building density is believed to be one of the fundamental parameters governing wave inundation and the maximum water heights. This experiment aims to find Manning’s friction values that can be used to replicate a similar hydrodynamic behaviour to specific building density values. The maximum water heights will be compared at t =1000 seconds, at which point both friction and building scenarios will have reached a ‘steady state’ situation. The steady state situation represents the maximum water heights that will be achieved for that particular scenario. The final water momentums past the buildings/friction blocks will also be checked to ensure the momentums/flow/velocities are similar to ensure a higher validity for the model.