SPOT Stakeholder Questionnaire

Current and Future Requirements

| SPOT Project | Digital Elevation Data & Model SPOT |
| --- | --- |
| Date | December 2006 |
| Stakeholder | Inundation Modelling Program (Risk Assessment Methods Project, Risk Research Group) |
| Division | GEMD |
| How Accessed | Delivered by National Mapping and Information Group, GEMD |
| Aim: To gather your requirements to determine a data model and governance framework for the SPOT dataset Things to think about: - What products are generated from this dataset? - Who are the potential users/clients? How do they use it? - What queries would you need? What frequency are they run? - What critical uses do you have for the data? - What international and national standards are relevant to this data? - What standards apply to the distribution of this data? For Example Requirement: Need to list out the wells in area <x> by company Details: Run this weekly to compile report. |  |

- Business Requirements

| Requirement | Brief Details |
| --- | --- |
| Business Driver | What business need is served by this dataset? Tsunami Risk Assessments for Australian coastal communities. |
| Client Requirements | Required for gap analysis and identifying process deficiencies. What availability expectations/requirements do clients have? What currency expectations/requirements do clients have? What distribution expectations/requirements do clients have? - Best available offshore and onshore data which has been Quality Assured. - Understanding of licence issues so that developed products can be released to government stakeholders. |
| Governance | What custodianship obligations exist? What governance roles exist and who/what fill these roles? We have no custodian obligations for the delivered data?? |

- Functional Requirements

| Requirement | Brief Details |
| --- | --- |
| Determine tsunami inundation extent for identified coastal communities | Generate model for identified community based on local topography and bathymetry to investigate tsunami inundation extent. Require topography and bathymetry to be; - QA’ed and logically consistent (“stitched”) - In same vertical and horizontal datum (GDA94 and AHD), - Best available resolution and accuracy, particularly in the coastal zone. We would prefer more accurate data over higher resolution, particularly in the coastal zone. - Metadata, including and understanding of accuracy Once data has been received, then a re-issue is only required if “better” data becomes available and external client requirement exists to re-simulate event. |
| Determine tsunami risk to identified coastal communities | As above, but simulate the model for a range of tsunami hazard events. |

- Are there any other Requirement Documents that are still Valid?

Where are they?

- Future Requirements

| Requirement | Brief Details |
| --- | --- |
| Delivery of multiresolution datasets | Delivery of a number of data sets so that models can adapt data at appropriate resolution. |
| Delivery of seamless datasets | Delivery of “stitched” offshore and onshore data sets |

- Do you have any of your data-related work processes documented?
If so please provide

i.e. Instructions or diagrams for data entry, QA, reporting etc.

e.g

  - Develop study extent and issue to NM&I and PMD for data sourcing (bathymetric data sourced from marine database)

  - Obtain QA’ed data from NM&I and store on project drive (IT&T backup)

  - Obtain tsunami event information and store on project drive (IT&T backup)

  - Build ANUGA model; input data, create mesh according to dataset, fit data to mesh, simulate model according to tsunami event

  - Store ANUGA scripts (model metadata), points set, generated mesh and model output on project drive (IT&T backup)

  - Validate data; export grid to NM&I to compare with original data supply

  - Once validated, continue model simulation and generate maximum inundation maps, time series information and visualisations (stored on project drive according to time of simulation)

  - Build decision support tool (stored on project drive)

  - Compile report describing outputs (stored on project drive and maintained through revision control system)

  - Distribute decision support tool, visualisation and report to State and Federal Government stakeholders (need to know licence issues for distribution of decision support tool).

  - Archive model data and outputs (Duncan – other things to say here?)

  - Model scripts and report maintained in revision control system.

  - Model scripts for particular simulations maintained with simulation output (not necessarily maintained in revision control system, but stored on project drive)

  - Distribution of report recorded in TRIM.

- Who can the project ask for more Information?

| Name | Ole Nielsen |
| --- | --- |
| Position | Lead, Inundation Modelling Program |
| Division | GEMD |
| Contact Number | x9048 |
| Alternative Contact | Duncan Gray, x9077 |