Open Source Storm Surge Modelling Software for Risk Analysis in the Making.

by Ole Nielsen and Duncan Gray.

"It is cheaper to run computer programs than to build models with concrete."

Staff within the Risk Research Group at Geoscience Australia are

developing software to simulate the impact of storm surges

along the Australian coastline. This is the initial step in assessing

the relative risks due to storm surges. It is also a general

simulator that can form the basis for a wide range of applications

such as studying, for example,

how a proposed break-water made of rocks and concrete

would protect a boat ramp from heavy swells under various conditions

and at different locations. At a larger scale the software will be capable

of modelling the effect of natural hazards such as storm surges or

riverine flooding.

In general, the software is intended to help decision makers at local,

regional, and national levels by providing a tool capable of

simulating scenarios that cannot otherwise be studied feasibly.

The software, which simulates the physics of water flow in rivers, estuaries

or along a coastline, is based on sophisticated mathematical and

numerical techniques developed by Dr Christopher Zoppou from GA and

Dr Stephen Roberts from the Australian National University. A state-of-the-art simulator based on these mathematical models has been designed and developed into an open source software package by Dr Ole Nielsen and Mr Duncan Gray.

In addition to the modelling of water flow, the mathematical modelling

approach is capable of simulating more general phenomena including

tsunamis, landslides and bomb blasts.

How does it work?

The fluid dynamics module, called Pyvolution, is based on a

finite-volume method for solving the Shallow Water Wave equation governing the behaviour of flowing water. The study area is represented by a large number of triangular cells and water depths and speeds are tracked over time by solving

the governing equation within each cell.

To set up a particular scenario the user of the system specifies

the geometry (bathymetry and topography), the initial water level,

what happens at the edge of the domain (called the boundary conditions)

such as the tide and any wind field that drives the water as well as

frictional resistance from various terrains.

The geometry could for example be a digital elevation

model of an estuary. To help with these tasks, a tool called Pmesh has

been developed. Pmesh allows the user to interactively set up the

geometry of the problem. It produces a triangular mesh of the study area

and allows the user to specify the locations and types of boundary conditions

that are going to be applied. Figure 1 <reservoir_pmesh.tif>

shows an example of a geometry generated by Pmesh.

To simulate, for example, the consequences of a water supply reservoir

breaking as the result of an earthquake, the topography surrounding

the reservoir is loaded into Pmesh.  The reservoir wall is

treated as a boundary condition. The nature of the reservoir collapse

and the initial depth of water in the reservoir are specified. The

generated mesh (106,185 triangular cells in this example) is passed to

Pyvolution, the simulator, which simulates the motion of

the water as it moves from cell to cell. Figure 2 <reservoir_pyvolution>

shows a visualisation of the dam break example. The water gushes out of

the breach, and driven by gravity finds its natural path downhill except

where it is diverted by an obstacle such as a simulated building.

Figure 2 is a screenshot from an animation tool which was also developed at

GA in collaboration with Dr Darran Edmundson from the ANU Supercomputing

Facility.

Capabilities

A major capability of Pyvolution is that it can model the process of

wetting and drying as water enters and leaves an area.  This means that it is suitable for

simulating water flow onto a beach or dry land and around structures such as buildings. It is also capable of capturing surges, commonly referred to as shocks by

mathematicians.  Think, for example, of a water from a tap splashing

water into a sink. There will typically be a flat area of water with

very shallow depths and high speeds surrounded by a thicker layer of

calmer water. The transition between the regions forms a shock. Shocks will be typically encountered in area of rapid changes in speed or bathymetry.

Figure 3 <patterns.tif> shows a simulation of water flowing through a

hypothetical breach in a levee, down onto a flat surface and past an

obstacle.  A number of complex patterns are captured in this example

including a shock where water reflected off the wall far (at the right

hand side) meets the main flow. Other physical features simulated are the

standing waves and interference patterns.

Software used

Most of the components are written in the programming language Python

which is an object oriented programming language known for it's

clarity, elegance, efficiency and reliability.  It is often said that

"Python lets you focus on the problem at hand".  This means that it is

possible to develop complex pieces of software without undue

distractions in dealing with idiosyncrasies of the software language

syntax. Consequently, software written in Python can be produced quickly

and can be readily adapted to changing requirements throughout its lifetime.

Python is freely available, is developed and supported by a large

number of organisations and comes with all source code available for

scrutiny and modifications.

As such Python is an example of Open Source Software (OSS) - a concept

which has been impacting every aspect of information technology over

the last few years.

The animation tool used with the storm surge model is based on OpenSceneGraph which is another OSS component allowing high level interaction with sophisticated graphics primitives.

The storm surge model will be released as OSS in year 2005. This strategy will enable free access to the software and allow the risk research community to easily use, validate and contribute to the development of the storm surge model.

###############################################################################

NOTE FOR THE EDITOR: Text from this point onwards (apart from the references) could form a break-out box explaining the concept.

###############################################################################

Open Source Software

Prominent examples of OSS are the Linux operating system (which is

today’s de facto Unix standard), the Apache web server (which

serves on about 65% of all web servers worldwide)

MapServer (which is a development environment for building spatially enabled Internet applications) and MySQL (A widely used relational database).

Examples of organisations who are using and contributing to OSS

include IBM, Sun, Hewlett Packard, Apple, Novell, Lawrence Livermore

National Laboratories, Google, Yahoo, Industrial Light and Magic and

many more.  More recently, governments have began to embrace open

source as a part of their software strategy both as a way to share

software they create and reduce costs. A recent example is the

city of Munich switching all 14,000 of its computers to the Linux

operating system.

In Australia, the ACT legislative assembly passed a bill in December 2003

(reference 1) that encourages the use of open source software in

recognition of the advantages mentioned above.

How does Open Source Software work?

The term "open source software" or OSS refers to software that can be

freely used, redistributed, analysed and modified by anyone. These

rights are backed by legally binding licenses such as the General

Public License (GPL) (see reference 2).

Development takes place in the form of projects

often involving several individuals, organisations and countries.

Open source software entails a new kind of competition, separated from

that of traditional business in that the products are generally not

owned by any single company and therefore cannot be purchased on the

market. See references 3 and 4 for more background on OSS.

The main advantages of Open Source Software are:

* The costs of using existing OSS components are almost zero. This has

obvious advantages for any organisation, as funds can be put to other

use. Equally important, tools can be assessed and selected quickly

given a number of alternatives.

* The software code is open and readable by anyone.  This means that

the software can be analysed, verified and modified by anyone with

programming skills, allowing for the addition of features needed by a

particular group.

Traditionally, there has been many wasted opportunities to re-use

perfectly viable code due to the absence of an accepted process and

legal framework. Open Source can address this issue.  If one

government agency is writing a system from scratch and making the

final product of such efforts available to other government

departments each organisation can leverage a large amount of software

and use it for their own purposes.

* Open source software draws its strength from people improving,

modifying or customising programs and giving them back to the open

source community so others can benefit from their work.  Additionally,

there is no notion of public liability with open source. Hence,

releasing open source code is low risk.

The large amount of quality open source software is made possible by

organisations whose main business isn’t production and sale of

commercial software. In fact, it is widely believed that only about 5%

of programs are made for the commercial market.  When software is not

the main business, but rather a means to an end, OSS offers a reliable

and affordable avenue as it has in the case of the storm surge

modelling software developed by the Risk Research Group at Geoscience Australia.

References

1) Government Procurement Guideline Act, clause 6A, passed on 11 December 2003:

http://www.legislation.act.gov.au/a/2003-63/20031219-10826/pdf/2003-63.pdf

2) Legal protection regarding the GPL OSS license:

http://www.gnu.org/philosophy/enforcing-gpl.html

3) Open Source Initiative: http://www.opensource.org

4) Open Source Industry Australia: http://www.osia.net.au/