The ANUGA Resource Experiment

Aim

To develop some code that will accurately predict the maximum run time of an ANUGA program given parameters from the code rathe than the user, so that this can be used to allow simulations to be run on NCI as well as giving users the ability to estimate how long a script will have to run for.

Method

(found in /trunk/anuga_work/development/mem_time_tests)

To ascertain an estimating equation for the hydrodynamic modelling program ANUGA, we want to see how the time taken is affected by just one variable, and so in these experiments the title of the experiment is what was changed over the course of multiple runs, so for instance the Percentage Of Map Covered By Water we are just changing how much of the map is covered by water, so in the first run we’ll have something like a completely dry map, and then in the second run (it loops automatically) it will be 10% of the map covered by water, etc. This results in data over many experiments with a different value for each variable (while the others did not change) and so one can see how the time taken is affected just by the variable itself. This further allows you to see which variables are important and how they relate example: linear, quadratic, log, exponential, etc. This then allows you to make an equation for the experiment, with place holding constants for all the variables examples: the log of the depth, the square of the number of processors, the number of file writes, etc. Once these variables are found one can run one experiment that changes just there variables and then perform a linear regression on them, resulting in an equation that estimates the resource you were looking at, specifically the time taken and the memory used in these experiments.

Nothing (/parameters/nothing)

This experiment used a sloped map of size 1000 by 500 (m) and a mesh matrix of size 100 by 300, with water flowing in from one side (but never leaving the experiment), and a final time of 40.0, with a time step of 0.2.

There were no variables except for the run number, the time and space of this experiment was recorded using the log analyser.

Number of File Writes (/parameters/timelen-over-timestep)

This experiment is on a flat map of size 1000 by 500 (m) and a mesh matrix of size 100 by 300, with a constant depth of still water(10m), and a final time of t, with a time step of delta.

The variable that was changed was the time step and the time length, the time and space of this experiment was recorded using the log analyser.

(Set-up:

For each n in range () – time length

For each m in range () – time step

Run code with these variables)

Time Length (/parameters/timelen)

This experiment is on a flat map of size 1000m by 500m and a mesh matrix of size 100 by 300, with a constant depth of still water (10m), and a final time of t, with a time step of t divided by 100 (to keep the number of file writes constant).

The variable that was changed was the time step and the time length, the time and space of this experiment was recorded using the log analyser.

Number Of Triangles (/triangles/fromregions)

This uses a flat map with 100m deep water flowing in one side at 223.52m/s and the other side being an outflow boundary. It uses the from regions method with a variable length of the side of the square map, and a variable maximum triangle area. This keeps the approximate area of the triangles constant (same maximum area) but changes the number of triangles. It has a final time of 2000 and a yield step of 120.

(Set-up:

For each n in range () – maximum triangle area

For each m in range () – map size

Run code with these variables)

Note: there is also a parallelised version of this. (/triangles/area)

Area of Triangles (/triangles/rectanglecross)

This uses a flat map with 100m deep water flowing in one side at 223.52m/s and the other side being an outflow boundary. It uses the rectangular cross method with a variable length of the side of the square map, and a variable matrix size for the triangles. This keeps the number of triangles constant (same matrix size) but changes the area of the triangles. It has a final time of 2000 and a yield step of 120.

(Set-up:

For each n in range () – matrix size

For each m in range () – map size

Run code with these variables)

Depth of Water/Extent (/scenarios/stage)

This map is flat with a variable extent, it has all reflective boundaries and a given water depth, no water moves throughout the experiment. The number of triangles is constant as it uses the rectangular cross method for the matrix (of size 100 by 300). It has a final time of 40.0 and a yield step of 0.2.

(Set-up:

For each n in range () – length of square side

For each m in range () – depth of water

Run code with these variables)

Velocity of Water (/scenarios/vel2)

This map is flat with a variable extent but a fixed number of triangles due to the use of the rectangular cross method (map size of 1000 by 500, and matrix of size 100 by 300). It has an inflow boundary at one end putting water into the system with the given velocity and an outflow boundary which takes water away. No water is present at the beginning of the simulation. It has a final step of 40.0 and a yield step of 0.2.

(Set-up:

For each n in range () – velocity

For each m in range () – map extent

Run code with these variables)

Percentage of map covered by water (/scenarios/channelflow)

This map is sloped with a step at a given point, so that only a known amount of map is covered by water during the experiment, water flows in from the top at 10m/s and flows out of the bottom at a depth shallower than the step. The map also is given a length and therefore an area. It uses the rectangular cross method with a fixed matrix size of 100 by 300. The friction is constant and the final time is 40.0 while the yield step is 0.2.

(Set-up:

For each n in range () – length of square side

For each m in range () – step placement

Run code with these variables)

n

m

Number of CPUs (/hardware/cairns)

In this experiment we take a simulation, make it parallel and then run it many times with different numbers of processes, recording the memory used, the time taken and the number of processes used. This was mainly done on the cyclone HPC with a quicker version of the run cairns script (smaller bounding polygon and lower resolution).

There is also a template script which is a lot like the normal experiment with from regions with a map size of 10000m by 10000m, a max triangle area of 100(m^2), a final time of 2000s, a yield step of 120s. With an initial water level of 100m, and water flowing into the experiment at 223.52m/s with the same depth. (/hardware/template)

Speed of CPUs (/parameters/nothing)

For this experiment the nothing script was run on Cyclone(normal node, and local node), Tornado(normal node, and local node) and Compute 1, the space, time and run number was recorded, and then these were compared to each other, to determine how the speed of CPUs affects the run time.

Major Variables (/linearregression/area)

In this experiment the 3 easiest to measure variables, the number of triangles, the time step and the final step were all recorded. For 30 experiments the number of file writes varied (ratio between time step and final step), then for 30 experiments the number of triangles changed (the triangle area was kept constant), then for 30 more experiments the final step was changed (number of file writes constant). For this experiment the velocity was 10m/s (36km/h), the depth of water was kept in ratio with the extent so that (depth/ extent = 0.0004), and the percentage of water was 100%, the friction was zero (it makes no difference anyway).

New Parallel (/linearregression/area)

In this experiment the standards from the major variables are used, and the only thing that is changed is the number of processors that the standard experiment is run on. Standard experiment:

Number of triangles: ~120,000

Final Time: 3600

Time step: 60

The rest are the same as the above experiment.

Results

Nothing (/parameters/nothing)

The time and space for this was constant, as would be expected form the same script running over and over.

the reason why this isn’t a perfect line is due to the experiment being slowed down hardware and node troubles, and doesn’t reflect a trend for the experiment itself.

We use the highest memory because it is the memory that most accurately reflects the memory usage that NCI records.

Number of File Writes (/parameters/timelen-over-timestep)

The time for this was linear, however the time length affected the gradients of the different lines, as the time length is changing and the affect of this is too push up the time taken resulting in graphs like this:

Here the different series represent the different time steps, for example blue diamonds are a time step of 9.1.

If you then graph the gradient of each line and graph those gradients against the time steps, you get this result:

This shows that this gradient affect only if the time steps are very small, otherwise it is pretty much constant, this is shown in the holistic graph of file writes against time taken:

This shows that although there isn’t one gradient, the gradient is pretty much the same so long as the time step is not small (less than 2 seconds).

For the memory usage it is much simpler and is just constant:

Time Length (/parameters/timelen)

The time taken is linear, as would be expected:

While the memory usage is constant:

Number Of Triangles (/triangles/fromregions)

Due to time constraints the final experiment of this was not fully run to a satisfactory level to get reliable data, so this has been omitted.

Area of Triangles (/triangles/rectanglecross)

Due to time constraints the final experiment of this was not fully run to a satisfactory level to get reliable data, so this has been omitted.

Depth of Water/Extent (/scenarios/stage)

The time for this is square root function, as can be seen by this graph of Depth over Extent vs. Time Taken:

Since this is linear, to get the time taken you need to take the square root of the Depth/Extent linear equation, meaning that this is a square root equation. This can also be seen in the sideways parabola drawn by the graph of Depth over Extent vs Time Taken.

While the memory use is again constant:

Velocity of Water (/scenarios/vel2)

The Time taken is linear, with extents not changing the gradient or the intercept for the linear equation between the velocity of the flowing water and the time taken:

Here the different series stand for different extents, for example an extent of 1 metre squared is represented by blue diamonds.

The fact that the gradients don’t change is shown here, (the intercepts change, but not with any particular trend)

the reason the intercepts are so different, is that changing the gradient just a little will cause the intercept to be changed so that it is the line of best fit, resulting in a large variance of data points, and so this graph is ignored in terms of significant relationships.

All the different velocities have constant memory use:

Percentage of map covered by water (/scenarios/channelflow)

The Time taken is not affected by how much of the map is covered by water, as seen here where percentage of water cover is graphed against time with different map sizes having different series on the graph:

Here the different series mean different side lengths of the map (which is square): for example a side length of 10 is represented by blue diamonds.

the reason that the smaller map slows down the simulation is that a lot of water will be flowing fast over the map, then the speed up is due to the water depth over extent being less, then the map size starts taking an effect slowing the time down again with the bottom cluster of series having larger maps higher and smaller maps lower.

The space is constant for all, as seen here:

Number of CPUs (/hardware/cairns)

Due to time constraints the final experiment of this was not fully run to a satisfactory level to get reliable data, so this has been omitted.

Speed of CPUs (/parameters/nothing)

Due to time constraints the final experiment of this was not fully run to a satisfactory level to get reliable data, so this has been omitted.

Major Variables (/linearregression/area)

This experiment was used to do linear regression on the multivariate data it produced, so here are the results of that linear regression (by least squares method):

Time = 0.0000395003931919 * (number of triangles squared) + 0.295751519887 * (number of triangles) + 0.0380473603309 * (time length) + 0.00592869267474 * (number of file writes) + -135.066117791 [with R squared = 0.990192607556]

Space = 0.00369571970894 * (number of triangles) + 331.71280946 [with R squared = 0.999953433992]

New Parallel (/linearregression/area)

Due to time constraints the final experiment of this was not fully run to a satisfactory level to get reliable data, so this has been omitted.

Conclusion

Nothing (/parameters/nothing)

Changing nothing has no effect on the memory used by the anuga script or the time taken to run the anuga script, as would be expected.

Number of File Writes (/parameters/timelen-over-timestep)

The Number of file writes certainly plays an important role (and is easy to measure) in relation to time, with this relationship being linear across different time lengths (provided the time step isn’t too small, which it won’t be for ‘standard’ anuga runs). This means we should include this parameter in our estimating equation. However it has no effect on the memory usage.

Time Length (/parameters/timelen)

The Final Time (one evolve parameter) has a linear relationship with time, and this is definitely going to be a major contributing factor to the time that anuga scripts takes to run. This is especially true because most ‘standard’ anuga scripts are going to run for a considerable amount of time, it is also easy to measure.

Number Of Triangles (/triangles/fromregions)

Although the final experiment was not fully run, all prior experiments pointed towards a quadratic equation for time and a linear one for space. With this information we just used those two equations in the major variables experiment, with the number of triangles and the number of triangles squared for time and the number of triangles for space. This resulted in a curve that fit the data of the major variables experiment well, affirming that these were the nature of the relationship between the number of triangles and the resource usage of the experiment.

Area of Triangles (/triangles/rectanglecross)

No full experiment of this was ever completed, and so it is unknown how without any other variable changing how the area of triangles affects the time and memory use of ANUGA. Although the graph of velocity with its constant number of triangles and changing map area indicates that this has no noticeable impact on the time taken and the memory used when running ANUGA.

Depth of Water/Extent (/scenarios/stage)

Ratio between the Depth of Water and the Extent is certainly important as can be seen in the results, however it is hard to ascertain how this variable would affect maps when it is not all the map that is covered, and also which point is better as input the average water depth of the deepest point. It is also hard to get any of these values until after the script is run, and so this is pointless. However it will need to be incorporated so that the estimation for time will at least over estimate the time taken in most cases. It won’t be included in the space equation, as it has no effect on the memory usage.

Velocity of Water (/scenarios/vel2)

The velocity of the water has a linear relationship with time (regardless of map size)

However like the depth of water over extent, it is hard to measure, hard to know which value to use. However it does play a big enough role in altering the time taken for the script to finish, so it would be best to include it as a constant, namely the speed at which  tsunamis travel through deep ocean about 223.52m/s. It has no effect on space, and so it does not need to be included in the final estimating equation.

Percentage of map covered by water (/scenarios/channelflow)

The percentage of water on the map has no effect on the time taken for the simulation to run and no effect on the memory used by the simulation.

Number of CPUs (/hardware/cairns)

All data on this is unreliable, and so no conclusions will be drawn about this experiment.

Speed of CPUs (/parameters/nothing)

There are 3 points of data for this experiment so far, and so no conclusions will be made about the nature of this relationship.

Major Variables (/linearregression/area)

The R squared values indicate that these equations are both very good equations to fit the results, and so these are both good bases on which to get the constants for the final estimating equation for a node on tornado.

New Parallel (/linearregression/area)

This experiment was never properly run.

Problems

The main problems with the estimating equation that I have produced are its simplification of the important variables of Depth over Extent and Velocity as well as its measure of Triangle Area and the Hardware it runs on.

In this equation we have chosen to have constants represent both the Depth over Extent and the Velocity, even though differing speeds and depths can have quite a major effect on the time it takes for the simulation to finish. This can be seen from the results for these experiments, with increases in velocity and depth having large effects on the time taken. Namely a 480% increase in time taken due to a doubling in the velocity (120m/s-240m/s) and a 300% increase in time taken due to a 900% increase in the ratio of depth over extent (0.002 1/m -0.018 1/m). For smaller Depths over Extents this increase is even more dramatic as the gradient of the curve tends towards infinity as the Depth over Extent tends towards 0. For this reason faster than expected moving water will results in the equation underestimating the time taken to complete the simulation, especially if these triangles are small, as fast moving water over small triangles causes the internal time steps to get smaller so that water doesn’t flow through the triangle in one time step. Deeper than expected water will also cause the time taken to be underestimated by the estimating equation as can be seen from the Depth over Extent vs. Time graph.

There is also a problem with how the triangle area was measured, as the maximum triangle area was used as a rough estimate for the triangle area. However, the simulation is more likely to be affected by the minimum triangle area, as small triangles will in general slow down the entire simulation especially if water is moving quite fast over them. For this reason the relationship between the minimum triangle area and the time taken is unknown and is therefore not included in the estimating equation, even though this seems to have quite a large effect on the time taken for the simulation to run.

The way the experiments are set up currently the results are not objective, and are skewed by the system the model is running on, and as current there is no quantified way that estimated how different systems will affect the resource usage of ANUGA. This is a serious issue, as ANUGA is intended as a cross platform program, and so any thing in it that relies on certain systems undermines a fundamental principle of the program. It also will result in equations incorrectly estimating resource usage because the tests have not been calibrated to the particular system that it is being run on

Recommendations

The simplest fix I would make would be to have the major variables experiment run again, but this time with the proper assumed values for velocity and depth over extent, namely 223.52m/s for velocity (the highest value it could be, it’s good to over estimate in this circumstance), and another value for depth over extent (I’m not sure of the general ratio between these two values, though it would be best if this over estimated it). With these two values over estimating the time taken it reduces the possibility of underestimating the time taken which is the worst outcome for this equation.

I would also work on the minimum triangle area experiment, as this effect will cause the estimating equation to underestimate the time taken, and so any work done to measure the effect and find the minimum triangle area would greatly increase the number of experiments that this estimating equation will either estimate correctly, or over estimate it thereby increasing the effectiveness of this estimating equation. In this I would also run the two triangle experiments area and number again (although smaller) so that the understanding of how the parameters of the triangles affect resource usage would be known.

I would also upon completing the other parts go on to investigate how differing systems affect the resource usage of the ANUGA model, so that the equation can more accurately estimate how long the simulation will have to run and how much memory it will need to run. This includes the parallel version of ANUGA and running it on multiple nodes, as this would allow the user to have a good guess of how many nodes would run their particular scenario in ANUGA optimally which would greatly increase the efficiency of the modelling process in ANUGA as do the relationships between user input variables and resource usage.